Lab 3.1.A: Putting opamps to ‘gainful’ use [Negative Feedback]

All question material is in blue. Please put your answers in black color font.
Note on symbols for Gain: 
In earlier labs we used the symbol G to denote the open-loop gain of an opamp. For the LM741 G~106(effectively infinite as approximated in most equations). 
As discussed in class, the opamp working in open-loop mode is not very useful. Our objective is to design and build an opamp circuit with a finite gain – we will call this gain Gf  i.e. the gain with feedback in place. Gf is a finite number whose value is specified by design

Part A: Simple Negative Feedback

A.1) Simple negative feedback to set finite voltage gain Gf

A.1.1 non-inverting negative feedback
In the pre-lab session, the following scheme of negative feedback was discussed to arrange a circuit with gain Gf=1x:

xVout

Fig 1: Basic scheme for opamp negative feedback, preserving the sign of Vin

A.1.1.1) Design												[2]

Arrange a resistor divider to feedback a fraction xVout of the output voltage Vout back to the V- input of the opamp.

Work out the design equations to calculate values of the resistor divider for Gf=+10 in your circuit design. Justify the choice of particular resistor values you use.  Write all steps of the design equation here: Use ‘Insert equation’ in MS-Word or Libre-Office to format your equations correctly including subscripts. A prototype equation is provided below, which you may copy-paste and reuse. One equation per line looks good!
Vout=G(V+-V-)

x=1G=110

Closed loop gain, G=1+R1R2

Therefore, R1R2=9

A decent choice of R1 and R2 that satisfies the above relation would be 6.8kΩ and 750Ω respectively.

A.1.1.2) LTSpice simulation

Draw your circuit diagram in LTSpice.									[2]
For clarity, only put your negative feedback opamp circuit design here (not full design with FG) 
You may use the standard opamp available in LTSpice OP07 as a stand-in replacement for LM741

Run the simulation in LTSpice and put below a plot of your simulated Vin → Vout signals. 
Now obviously you need to include the FG simulation for this step.

Remember to put large value electrolytic (100μF/47μF/22μF/10μF) power supply bypass capacitors for both FG and opamp power rails. Note that you will be running the opamp with ≈ ±8V VCC rail voltage as setup in Lab 2. From the simulation determine a suitable amplitude of Vin such that Vout does not hit the opamp saturation voltage limits when you test the circuit experimentally in the next step.Objective of your simulation: Verify the gain and phase difference (if any) in the transfer function Vin→Vout

To check the phase difference, you must  display both Vin and Vout with individual scaling on Y-axis: First plot just Vin using the voltage probe tool. Then, select the Output window and choose “Add Plot Pane” option from the “Plot Settings” drop-down control. Next, plot Vout as the second voltage using the voltage probe tool										     [2]

A.2) Circuit demo: Non-inverting amplifier with Vout=+10Vin		[6]

Build the opamp circuit as designed and simulated in the above steps on your breadboard. Drive the input Vin with the FG used in earlier labs. Set the FG output amplitude as chosen in the simulation.

Put a photo of your working setup here, clearly labelling Vin and Vout. With a single channel DSO measurement, it is not possible to measure the phase difference in experiment (at least at this stage, maybe later?!)

Vin

Vout

A.3) Negative feedback, inverting the sign of Vin

By a rearrangement of node connections in Fig 1 it is possible to apply a negative sign to the gain:      Vout=-GfVin.
This is called an ‘inverting’ configuration since Vout is out of phase with respect to Vin by 180° 
Keep in mind that for negative feedback, by definition a fraction of Vout must be applied to the V- terminal of the opamp. However, you are free to cleverly connect  Vin to either  V+ or  V-

Work out your circuit design and component values for implementing the experimental relation  Vout=-10×Vin : the absolute value of the  Gf is the same, but now there is a negative sign!

A.3.1)													[4]

Circuit design:		2 marks

Equations:			1 marks

LTSpice simulation:	1 marks

Applying KCL at node x,

Vin-0R1=0-VoutR2

Therefore, VoutVin= -R2R1

If we want the gain to be -10, then R1 and R2 can be taken as 680Ω and 6.8kΩ respectively.

A.3.2) Circuit demo of inverting amplification: Vout=-10×Vin 		[3]

Build the circuit for inverting amplifier as per A.3.1 and put photos of your working setup.

Vin

Vout

A.3.3) With only a single channel measurement on the DSO, how can you tell that the amplifier is inverting the sign of  Vin? It IS possible!								[3]

(Check the input amplitude Vin being sent from your FG to opamp, and look ahead to question A.4 for a hint)

One way to identify this is the fact that V+ will almost be equal to V- in this case. Hence if we connect the black probe to V+ (ground), the reading should be ~0V when the red probe is connected to V-. Also, since V- is approx. 0V and that the feedback is negative, the output voltage will be negative wrt V-, therefore inverted wrt to Vin.

Another way is to use an “asymmetric” input such as a square wave, say between 0 and 9V. Then the output voltage will range between 0 and a negative number. This will confirm that the output is inverted wrt the input.

A.4) Input impedances?

1) What is the input impedance of the  straightforward “non-inverting” configuration of A.1.1, when Vout=+GfVin?										[2]

Explain your answer with calculation steps & logical reasoning of the flow of current into/out of various nodes (you can check this in your LTSpice simulation)

The input impedance in this case is generally very high

We have the relation, Zin=1+G0GfZ0, where Gf is the closed loop gain, G0 is the open loop gain and Z0 is the input impedance without any feedback.

In this case, Zin = Vin/I. Now since I is extremely small, Zin will be very large. Almost all the current passing through R1 will go to R2.

2) What is the input impedance of the “inverting” configuration of A.1.2, when 
Vout=-GfVin?												[2]

Explain your answer with calculation steps & logical reasoning of the flow of current into/out of various nodes (you can check this in your LTSpice simulation)

In order that the circuit can operate correctly, the difference between the inverting and non-inverting inputs must be very small - the gain of the op amp is very high and therefore for a small output voltage, the difference between the two inputs is small.

This means that inverting input must be at virtually the same potential as the non-inverting one, i.e. at ground. As a result the input impedance of this op amp circuit is equal to the resistor R2 (the input resistance).