Lab 3.2: Putting opamps to ‘gainful’ use [POSITIVE Feedback]

Conceptual introduction

The prototype design for an opamp circuit using positive feedback is shown below in Fig 1

Fig 1: Prototype showing essential features of a positive feedback circuit. Fraction xVout is send as feedback to the V+ terminal

xVout

The two defining features of a positive feedback circuit are:

1) feedback signal is always applied to the V+ terminal

2) Vout can have only two values - +Vsat or -Vsat : these are the two states of the output – we will refer to the output ‘switching’ between these two states. Vsat is the maximum saturation voltage of the opamp output, usually a volt or so less than VCC

The two Golden Rules of opamp design are obviously still applicable and all analysis is done on the basis of those two rules.

1) Circuit design

Problem statement: 
What we DON’T want:  Opamp running open-loop acts a comparator. 
In Fig 1, consider what happens when the circuit in Fig 1 does not have the feedback loop in place. If x=0 ( i.e. V+ is set to GND (and disconnected from Vout, breaking the feedback connection) then, Vout will switch states when Vin transitions from Vin<0 to Vin>0 and vice versa. However, as we know voltages are always noisy! So a small noise fluctuation in Vin around transition threshold will cause the output to switch multiple times. 
With your kit’s current battery state (maybe much less than 9V!) check the Vsat values you get with opamp open-loop comparator – most of the calculations below will depend on the numerimcal Vsat

WHAT WE WANT: Design a circuit that is called a “Schmitt Trigger” 
The problem with noisy Vin and the desired solution for behavior of Vout is shown in Fig 2

Fig 2: Vout of open loop comparator switches many times as noisy Vin crosses threshold. 
OBJECTIVE: Should be able to set a specified value of threshold voltage  VT and have a specified safety ‘noise’ band around the threshold (marked as blue shaded box): 
Required circuit:  Vout must switch only when Vinexits that safety band.
Note: Timing & logic of the transitions highlighted above are caused by the intrinsic structure of our inverting Schmitt trigger.

Design specification:

As per Figures 1 and 2, design a positive feedback circuit using the LM741 such that:

- Threshold Voltage VT=2V

- Noise band allowed around VT   is±0.3V

- Use regular opamp power supply setup as earlier ±VCC~±8V don’t forget the power supply bypass caps!

Hints for design:

- As in negative feedback the fraction xVout is set by a resistor divider. However, now our analysis is constrained by Vout being stuck to ±Vsat  and having to set two bounds on the comparison : 
both 2V+0.3V and 2V-0.3V
For simplicity we have asked for a symmetrical noise band around VT. In principle it is possible to set different lower and upper bounds around VT

- You will find THREE resistors are required instead of two as used in the negative feedback case.

- Generally, such a circuit is called a ‘Schmitt Trigger’

Solution to Question 1) 										5

Put a sketch of your Schmitt Trigger design here labelling the resistors and the design calculation equations that fulfill the required specifications.

(can be photo of hand-drawn sketch). Equations must be properly formatted

We have, VT=RR3VCC±RR1Vsat

Where R = R1||R2||R3 and Vcc = Vsat = 8V

We equate RR3VCC=2V and RR1Vsat=0.3V

On solving these, we get R1=203R3 and R2=2057R3

R3 can be chosen to be 6.8kΩ. Therefore, R1 = 47kΩ and R2 = 2.2kΩ

2) Simulation											5

Simulate the circuit in LTSpice. For Vin Use the full-scale output of your standard square waveform from FG’s collector of transistor Q3. Normally, the square wave output of the FG swings 0V to slightly less than VCC. 
Simulation will give a very fast transition across the threshold voltage. But note that in the next question, when you do the circuit test the actual VCC may be much lesser, and Vin will be quite a bit noisier!

Circuit diagram			2 marks

Plot of Vout & V+ (time domain)	1 mark

Plot of Vout v/s Vin 			2 marks (notice the hysteresis behavior!)

Circuit Diagram

Vout vs time

V+ vs time

Vout vs Vin

3) Circuit test – electrical voltages							5

Build the Schmitt trigger circuit designed and simulated on your breadboard.

Do a photogenic demo of your V+ and the Vout of your Schmitt trigger. Label each photo to distinguish which is which.
Use the full scale square wave output from FG as Vin

Vout					V+

4) Circuit test – noisy light intensity measurement			10

Now we would like to test the Schmitt trigger with a realistic, noisy sensor voltage input.

Sensor:  Setup the photo-transistor in your kit as a light detector. With the base left unconnected, the voltage obtained at the emitter is proportional to the light incident on the device

Breadboard

LED

SENSOR

60°

60°

Ambient light introduces noise!

Light source:  Use an LED connected to the FG output with a suitable series resistor to provide a variable light intensity to your sensor photo-transistor. Note that the LED and photo-transistor must be mounted facing each other on the breadboard as close to each other as possible. The LED has ≈ 60° cone of light emission and the phototransistor also has maximum sensitivity within a 60° cone as shown below.

(next page)

Vin							Vout

We observe that the voltage waveform from the phototransistor happens to be a sawtooth wave. The output waveform from the op amp due to the Schmitt trigger will be a square wave (as shown in the right figure).